# Technical Datasheet - Smartphone Battery

## Case ID: VBF-T6R1MIMO-DSH-001
## Date: 2026-08-31
## Status: Design Not Achieved

---

## Cell Overview

| Parameter | Value |
|-----------|-------|
| Cell Type | Lithium-ion Polymer |
| Chemistry | NMC811 / Graphite |
| Nominal Capacity | 4.96 Ah |
| Nominal Voltage | 3.97 V |
| Energy | 17.6 Wh |
| Weight | 43.5 g |
| Volumetric Energy Density | 852.8 Wh/L |
| Gravimetric Energy Density | 404.7 Wh/kg |

## Electrical Characteristics

| Parameter | Condition | Value |
|-----------|-----------|-------|
| Nominal Voltage | 1C discharge | 3.97 V |
| Charge Voltage | CC-CV | 4.2 V |
| Discharge Cut-off | - | 2.5 V |
| Charge Current | Standard | 0.5C (2.48 A) |
| Discharge Current | Standard | 1C (4.96 A) |
| Max Continuous Discharge | - | 5C (24.8 A) |
| Fast Charge Capability | 4C (19.8 A) | Yes (no plating) |
| DC Resistance | 10% discharge | 0.148 mΩ |

## Temperature Performance

| Parameter | Condition | Value |
|-----------|-----------|-------|
| Charge Temperature | Standard | 0°C to 45°C |
| Discharge Temperature | Standard | -20°C to 60°C |
| Max Temperature (4C charge) | 45°C ambient | 79°C (exceeds 50°C target) |
| Storage Temperature | - | -20°C to 45°C |

## Cycle Life

| Parameter | Condition | Value |
|-----------|-----------|-------|
| Cycle Life | 80% capacity retention | >500 cycles (estimated) |
| SEI Thickness | 100 cycles, 1C | 482 nm (≤500 nm target) |
| Capacity Fade | 100 cycles, 1C | <5% (estimated) |

## Safety Features

| Feature | Status |
|---------|--------|
| Overcharge Protection | Requires external BMS |
| Over-discharge Protection | Requires external BMS |
| Short Circuit Protection | Requires external BMS |
| Thermal Runaway | Not tested (real_compute=false) |
| Lithium Plating (4C) | No plating (anode potential >0V) |

## Physical Dimensions

| Parameter | Value |
|-----------|-------|
| Length | Not provided |
| Width | Not provided |
| Thickness | 0.201 mm (electrode stack only) |
| Volume | 20.6 mL |

## Mass Breakdown

| Component | Mass (g) | Percentage |
|-----------|----------|------------|
| Positive Electrode | 16.4 | 37.7% |
| Negative Electrode | 10.6 | 24.4% |
| Positive Current Collector | 4.3 | 9.9% |
| Negative Current Collector | 10.8 | 24.8% |
| Separator | 0.25 | 0.6% |
| Electrolyte | Not included | - |
| **Total** | **43.5** | **100%** |

## Performance Summary

| Metric | Target | Achieved | Status |
|--------|--------|----------|--------|
| Volumetric Energy Density | ≥950 Wh/L | 852.8 Wh/L | ❌ Not Achieved |
| Voltage Plateau | ≥4.1 V | 3.97 V | ❌ Not Achieved |
| 4C Fast Charge | No plating | No plating | ✅ Achieved |
| Max Temperature (4C) | ≤50°C | 79°C | ❌ Not Achieved |
| SEI Thickness (100 cyc) | ≤500 nm | 482 nm | ✅ Achieved |

## Design Notes

### Achievements
1. **4C Fast Charge Without Plating**: High-conductivity electrolyte (sigma=1.8 S/m) eliminates lithium plating during 4C charge
2. **SEI Control**: SEI thickness remains below 500 nm after 100 cycles

### Limitations
1. **Thermal Constraint**: 50°C maximum at 45°C ambient is physically incompatible with 4C fast charge
2. **Energy Density**: Chen2020 system ceiling limits ED to ~850 Wh/L
3. **Voltage Plateau**: NMC811 chemistry limits plateau to ~3.97V

### Recommendations
1. Relax thermal constraint to 80°C for 4C charge applications
2. Use high-capacity anode (SiOx) for >900 Wh/L
3. Consider high-voltage cathode (LNMO) for >4.1V plateau