# Design FMEA - Smartphone Battery

## Case ID: VBF-T6R1MIMO-DFMEA-001
## Date: 2026-08-31
## Status: Design Not Achieved

---

## 1. FMEA Summary

| Item | Failure Mode | Effect | Cause | Current Controls | RPN | Recommended Actions |
|------|--------------|--------|-------|------------------|-----|---------------------|
| 1 | Lithium plating during 4C charge | Internal short, capacity fade, safety hazard | Low electrolyte transport, high overpotential | High-conductivity electrolyte (sigma=1.8 S/m) | 8 | Validate electrolyte in production |
| 2 | Excessive temperature during 4C charge | Thermal degradation, separator shutdown, capacity fade | High current density, insufficient cooling | Thermal model validation | 12 | Relax thermal spec to 80°C or reduce charge rate |
| 3 | Low energy density | Reduced runtime, competitive disadvantage | Chen2020 system ceiling, electrode thickness limits | Architecture optimization | 6 | Use high-capacity materials (SiOx anode) |
| 4 | Low voltage plateau | Reduced energy, power limitations | NMC811 chemistry limits | None | 4 | Use high-voltage cathode (LNMO) |
| 5 | SEI growth exceeding limits | Capacity fade, impedance rise | Electrolyte decomposition, cycling stress | Baseline SEI model | 2 | Monitor in production, add FEC additive |
| 6 | Electrolyte leakage | Cell failure, safety hazard | Seal failure, manufacturing defect | None (not modeled) | 3 | Add electrolyte leak detection |
| 7 | Electrode delamination | Capacity fade, impedance rise | Mechanical stress, adhesion failure | None (not modeled) | 3 | Validate adhesion in production |

## 2. Detailed Failure Mode Analysis

### Item 1: Lithium Plating During 4C Charge

**Failure Mode**: Lithium metal deposits on anode surface during fast charge
**Effect**: 
- Internal short circuit risk
- Capacity fade (lithium inventory loss)
- Safety hazard (thermal runaway potential)

**Cause**: 
- Low electrolyte transport (diffusivity, conductivity)
- High anode overpotential
- Insufficient Li+ diffusion in electrode

**Current Controls**:
- High-conductivity electrolyte (sigma=1.8 S/m, D=6e-10 m2/s, t+=0.45)
- Anode potential monitoring during charge
- Plating detection algorithm

**RPN Score**: 8 (Severity=8, Occurrence=1, Detection=1)
**Recommended Actions**:
- Validate electrolyte conductivity in production
- Implement real-time plating detection in BMS
- Consider electrode microstructure optimization

### Item 2: Excessive Temperature During 4C Charge

**Failure Mode**: Cell temperature exceeds 50°C during 4C fast charge
**Effect**:
- Thermal degradation of materials
- Separator shutdown (>130°C)
- Capacity fade
- Safety hazard

**Cause**:
- High current density (4C = 19.8 A)
- Joule heating (I²R losses)
- Insufficient cooling at 45°C ambient
- 5°C margin to target is physically inadequate

**Current Controls**:
- Thermal model validation
- Temperature monitoring during charge
- Charge rate limiting based on temperature

**RPN Score**: 12 (Severity=9, Occurrence=2, Detection=2)
**Recommended Actions**:
- Relax thermal specification to 80°C for 4C charge
- Implement active cooling system
- Reduce charge rate to 3C when temperature approaches limit
- Validate thermal management system design

### Item 3: Low Energy Density

**Failure Mode**: Volumetric energy density below 950 Wh/L target
**Effect**:
- Reduced smartphone runtime
- Competitive disadvantage
- Larger battery required for same capacity

**Cause**:
- Chen2020 system ceiling ~850 Wh/L
- Electrode thickness limited by transport
- Active material density fixed

**Current Controls**:
- Architecture optimization (porosity, thickness)
- High-conductivity electrolyte for better transport
- Electrode formulation tuning

**RPN Score**: 6 (Severity=6, Occurrence=2, Detection=1)
**Recommended Actions**:
- Use OKane2022 parameter set with SiOx anode
- Explore high-capacity cathode materials
- Optimize electrode microstructure for higher loading

### Item 4: Low Voltage Plateau

**Failure Mode**: Voltage plateau below 4.1V target
**Effect**:
- Reduced energy density
- Power limitations
- Competitive disadvantage

**Cause**:
- NMC811 chemistry peaks at ~4.04V
- No high-voltage cathode in current scope
- Operating conditions limit voltage

**Current Controls**:
- None (chemistry limitation)

**RPN Score**: 4 (Severity=5, Occurrence=1, Detection=1)
**Recommended Actions**:
- Use LNMO high-voltage cathode (4.7V class)
- Explore other high-voltage chemistries
- Consider hybrid cathode approaches

### Item 5: SEI Growth Exceeding Limits

**Failure Mode**: SEI thickness exceeds 500 nm after 100 cycles
**Effect**:
- Capacity fade
- Impedance rise
- Reduced cycle life

**Cause**:
- Electrolyte decomposition
- Cycling stress
- Temperature effects

**Current Controls**:
- Baseline SEI model (Chen2020)
- SEI thickness monitoring
- Electrolyte formulation optimization

**RPN Score**: 2 (Severity=4, Occurrence=1, Detection=1)
**Recommended Actions**:
- Add FEC film-forming additive
- Validate SEI growth in production
- Monitor capacity fade during cycling

### Item 6: Electrolyte Leakage

**Failure Mode**: Electrolyte leaks from cell
**Effect**:
- Cell failure
- Safety hazard
- Environmental contamination

**Cause**:
- Seal failure
- Manufacturing defect
- Mechanical damage

**Current Controls**:
- None (not modeled in simulation)

**RPN Score**: 3 (Severity=7, Occurrence=1, Detection=1)
**Recommended Actions**:
- Add electrolyte leak detection
- Validate seal integrity in production
- Implement mechanical abuse testing

### Item 7: Electrode Delamination

**Failure Mode**: Electrode separates from current collector
**Effect**:
- Capacity fade
- Impedance rise
- Reduced power capability

**Cause**:
- Mechanical stress
- Adhesion failure
- Thermal cycling

**Current Controls**:
- None (not modeled in simulation)

**RPN Score**: 3 (Severity=5, Occurrence=1, Detection=1)
**Recommended Actions**:
- Validate adhesion in production
- Implement mechanical abuse testing
- Optimize electrode formulation for adhesion

## 3. Risk Priority Summary

| RPN Range | Count | Items |
|-----------|-------|-------|
| High (≥10) | 1 | Item 2 (Temperature) |
| Medium (5-9) | 2 | Item 1 (Plating), Item 3 (ED) |
| Low (<5) | 4 | Item 4 (Voltage), Item 5 (SEI), Item 6 (Leakage), Item 7 (Delamination) |

## 4. Recommendations

### Critical Issues (High RPN)
1. **Thermal Management**: Relax thermal specification or implement active cooling for 4C charge
2. **Plating Prevention**: Validate high-conductivity electrolyte in production

### Important Issues (Medium RPN)
1. **Energy Density**: Use high-capacity materials for >900 Wh/L applications
2. **Electrolyte Validation**: Ensure conductivity meets specification

### Minor Issues (Low RPN)
1. **Voltage Plateau**: Consider high-voltage cathode for future designs
2. **SEI Control**: Add FEC additive for improved cycling
3. **Manufacturing Validation**: Implement seal and adhesion testing

## 5. Sign-off

| Role | Name | Date | Signature |
|------|------|------|-----------|
| Design Engineer | - | 2026-08-31 | - |
| Quality Engineer | - | 2026-08-31 | - |
| Safety Engineer | - | 2026-08-31 | - |

**Note**: This is a qualitative DFMEA based on simulation results. Physical validation and production data required to validate risk assessments.