# Design Verification Report (DVP&R)

## Case ID: VBF-T6R1MIMO-DVPR-001
## Date: 2026-08-31
## Status: Design Not Achieved

---

## 1. Test Summary

| Test ID | Test Description | Target | Result | Pass/Fail |
|---------|------------------|--------|--------|-----------|
| DV-001 | 1C Discharge Capacity | ≥4.9 Ah | 4.96 Ah | ✅ PASS |
| DV-002 | Volumetric Energy Density | ≥950 Wh/L | 852.8 Wh/L | ❌ FAIL |
| DV-003 | Voltage Plateau | ≥4.1 V | 3.97 V | ❌ FAIL |
| DV-004 | 4C Fast Charge (No Plating) | No plating | No plating | ✅ PASS |
| DV-005 | Maximum Temperature (4C) | ≤50°C | 79°C | ❌ FAIL |
| DV-006 | SEI Thickness (100 cycles) | ≤500 nm | 482 nm | ✅ PASS |
| DV-007 | DC Resistance | - | 0.148 mΩ | - |

## 2. Detailed Test Results

### DV-001: 1C Discharge Capacity
- **Test Condition**: 1C constant current discharge, 25°C
- **Target**: ≥4.9 Ah
- **Result**: 4.96 Ah
- **Pass/Fail**: ✅ PASS
- **Source**: elec_e_1c_discharge.json:capacity_ah
- **Notes**: Baseline capacity meets requirement

### DV-002: Volumetric Energy Density
- **Test Condition**: Calculated from 1C discharge energy and cell volume
- **Target**: ≥950 Wh/L
- **Result**: 852.8 Wh/L
- **Pass/Fail**: ❌ FAIL
- **Source**: elec_e_energy.json:energy_density_wh_l
- **Notes**: 10% short of target. Chen2020 system ceiling ~850 Wh/L.

### DV-003: Voltage Plateau
- **Test Condition**: Midpoint voltage during 1C discharge
- **Target**: ≥4.1 V
- **Result**: 3.97 V
- **Pass/Fail**: ❌ FAIL
- **Source**: elec_e_energy.json:midpoint_voltage_v
- **Notes**: 0.8% short of target. NMC811 peaks at ~4.04V.

### DV-004: 4C Fast Charge (No Plating)
- **Test Condition**: 4C charge at 45°C, thermal coupled, plating module enabled
- **Target**: No lithium plating (anode potential >0V)
- **Result**: No plating (anode potential min = 0.018V > 0)
- **Pass/Fail**: ✅ PASS
- **Source**: elec_e_4c_charge.json:anode_potential_v
- **Notes**: High-conductivity electrolyte (sigma=1.8 S/m) eliminates plating

### DV-005: Maximum Temperature (4C)
- **Test Condition**: 4C charge at 45°C, thermal coupled
- **Target**: ≤50°C (323.15 K)
- **Result**: 79°C (352.17 K)
- **Pass/Fail**: ❌ FAIL
- **Source**: elec_e_4c_charge.json:T_max_K
- **Notes**: Exceeds target by 29°C. 4C charge generates ~34°C temperature rise. Thermal constraint physically incompatible with 4C at 45°C ambient.

### DV-006: SEI Thickness (100 cycles)
- **Test Condition**: 1C charge/discharge for 100 cycles, 25°C
- **Target**: ≤500 nm
- **Result**: 482 nm
- **Pass/Fail**: ✅ PASS
- **Source**: elec_e_aging_100cyc.json:sei_thickness_nm_end
- **Notes**: Meets target with margin

### DV-007: DC Resistance
- **Test Condition**: Calculated from 1C discharge curve
- **Target**: Not specified
- **Result**: 0.148 mΩ
- **Pass/Fail**: - (no target)
- **Source**: elec_e_energy.json:dcr_ohm
- **Notes**: Excellent low resistance due to high-conductivity electrolyte

## 3. Failure Analysis

### DV-002: Volumetric Energy Density
- **Root Cause**: Chen2020 system ED ceiling ~850 Wh/L
- **Contributing Factors**: 
  - Electrode thickness limited by transport requirements
  - Active material density fixed by parameter set
- **Recommendation**: Use OKane2022 parameter set with SiOx anode for higher capacity

### DV-003: Voltage Plateau
- **Root Cause**: NMC811 chemistry peaks at ~4.04V
- **Contributing Factors**:
  - Cathode OCP curve fixed by parameter set
  - No high-voltage cathode option in current scope
- **Recommendation**: Use LNMO high-voltage cathode (4.7V class)

### DV-005: Maximum Temperature
- **Root Cause**: 4C charge generates ~34°C temperature rise
- **Contributing Factors**:
  - High current (4C = 19.8 A) generates significant Joule heating
  - 45°C ambient leaves only 5°C margin to 50°C target
  - Thermal model correctly predicts temperature rise
- **Recommendation**: Relax thermal constraint to 80°C for 4C charge applications

## 4. Test Conclusions

### Achievements
1. **4C Fast Charge Without Plating**: High-conductivity electrolyte successfully eliminates lithium plating
2. **SEI Control**: SEI growth remains within limits after 100 cycles
3. **Low DC Resistance**: Excellent transport properties achieved

### Limitations
1. **Thermal Constraint**: 50°C at 45°C ambient is physically incompatible with 4C charge
2. **Energy Density**: System ceiling limits ED to ~850 Wh/L
3. **Voltage Plateau**: Chemistry limits plateau to ~3.97V

### Recommendations for Production
1. **Relax thermal specification** to 80°C maximum for 4C charge applications
2. **Use high-capacity materials** (SiOx anode) for >900 Wh/L applications
3. **Consider high-voltage cathode** (LNMO) for >4.1V plateau requirements
4. **Validate thermal management** system can handle 79°C during fast charge

## 5. Sign-off

| Role | Name | Date | Signature |
|------|------|------|-----------|
| Design Engineer | - | 2026-08-31 | - |
| Test Engineer | - | 2026-08-31 | - |
| Quality Engineer | - | 2026-08-31 | - |
| Program Manager | - | 2026-08-31 | - |

**Note**: This is a virtual test report based on simulation results. Physical validation required before production release.