# Technical Datasheet — T8-40g

Document code: VBF-T8R1-DSH-01  |  Case: t8_r1  |  Date: 2026-08-25

Prepared: ________  Reviewed: ________  Approved: ________

| Field | Value |
| --- | --- |
| Rated capacity (Ah) | Nominal 6.1494 (parameter set, scaled); verified 6.9427 (1C DFN simulation) |
| Nominal voltage / voltage window (V) | Window 2.5 – 4.2; discharge midpoint 3.876 (parameter set / simulation) |
| Rated energy (Wh) | 25.246 (time integration of V·I, 1C discharge) |
| Energy density (Wh/kg) | 634.32 (contract-caliber mass formula; electrolyte excluded — annotated) |
| Energy density (Wh/L) | 1027.1 (stack volume caliber) |
| Maximum continuous discharge rate | 5C verified: 6.8571 Ah, retention 98.77%, T_max 319.99 K (DFN, 25 °C ambient) |
| Fast-charge capability | 4C @45 °C: T_max 330.41 K (≤ 333.15 K), no plating (anode min +0.0082 V); acceptance voltage-limited to 0.94 Ah — 4C for top-up only, ≤2C for full charge |
| Operating temperature range | 25 °C discharge / 45 °C charge simulated; other temperatures not simulated (Not provided) |
| Cycle life | Not simulated (requires aging model) — not fabricated |
| Safety determination | 4C-45 °C temperature rise PASS; plating PASS (protocol defaults; see DVPR) |
| DC internal resistance / power density | 0.684 mΩ / 156.1 kW/kg (mechanical derivation) |
| Dimensions | 72.09 × 1752.2 × 0.195 mm (unwound electrode geometry; shell thickness not provided) |
| Mass (g) | 39.800 (contract caliber, electrolyte excluded; +11.03 g electrolyte at lit. density 1.2 g/cm³) |