# Technical Datasheet - VBF-T8R2-DSH-01

Case t8_r2, final design V_G (Chen2020 NMC811/graphite). All values from parameter set / tool outputs; source per line. Not simulated or not provided items are stated honestly.

| Rated capacity (Ah) | Nominal 5.000 (parameter set); simulated 1C DFN 5.0351 (cell/r3_VG_1c_dfn.json:capacity_ah) |
| --- | --- |
| Nominal voltage / window (V) | 2.5 - 4.2 (parameter set Lower/Upper voltage cut-off); midpoint voltage 3.9029 (cell/r3_VG_energy.json:midpoint_voltage_v) |
| Rated energy (Wh) | 18.229 (discharge energy, time integral of V*I_1C; cell/r3_VG_energy.json:energy_wh) |
| Energy density (Wh/kg) | 483.4 (cell/r3_VG_energy.json:energy_density_wh_kg, contract formula: energy_wh / mass_kg; mass = sum layer thickness x (1-porosity) x density x area) |
| Energy density (Wh/L) | 935.3 (cell/r3_VG_energy.json:energy_density_wh_l; volume = stack thickness x area) |
| Maximum continuous discharge rate | 5C verified: capacity 4.8492 Ah, retention 0.9631 vs 1C (contract target >= 0.90 at 5C; DFN, cell/r3_VG_5c_dfn.json / cell/r3_VG_1c_dfn.json) |
| Fast-charge capability | 4C charge at 45 C: T_max 351.93 K (78.8 C, +33.78 K over ambient); no plating (anode potential min 0.0215 V > 0; cell/r3_VG_safety.json, protocol 4C_charge_45C, thermal lumped + plating) |
| Operating temperature range | Simulated conditions only: discharge at default ambient (reference temp 298.15 K, T_max 306.15 K at 1C); 4C fast charge at 45 C ambient (protocol). Full envelope not characterized - honest scope note. |
| Cycle life | Not simulated (aging model not exercised; task contract has no cycle-life criterion) - not fabricated |
| DCR (informational) | 2.7305 mOhm, derived by calc-energy as (V(t=0) - V(t=10%))/I_1C from the 1C curve (cell/r3_VG_energy.json:dcr_ohm). Pulse-based DCR: N/A (requires physical experiment). |
| Safety determination | 4C/45C charge: no plating (PASS vs contract plated=false); T_max record 351.93 K (no contract threshold) |
| Dimensions and mass | stack thickness 150.6 um; electrode height 65.0 mm x width 1990.8 mm; mass 37.71 g (contract caliber, electrolyte excluded), annotated electrolyte 6.40 g |